How to add an extension or a telephone number

Select Generic SIP Device and click on submit

Type user extension 	:	3650

Display Name		:	CI School

Go to under the dialog box chose secret option for password

Then click on submit

Click apply configuration changes here .New extension has been created.

Configure a software phone (Zoiper)

Click Here for Option

Click Add new SIP account

Put the name here like (CI school) and click OK or any other nam

Domain /Server IP	: 	192.168.55.1

User name		:	3650

Password/Secret	:	3650sip

Click on Apply

Click on OK

Now the software phone is ready to use.

Click here for dial pad

Backup and Restore

Go tosystem      Backup/Restore

Click on Perform a Backup

Select all options and click on Process

New backup is completed

To check new backup go to Backup/Restore.

You can see the following page.

You can download the backup file by clicking here

Restore

Select the backup file (which file you want to restore) and click on Restor

Trunks

Click on System-Hardware detector (to check the digium card)

Click PBX-Trunks-Add DAHDI Trunk

Fill up the following info as below

Trunk name		:

Match pattern		:     .

DAHDI Identifier	:

Now Submit Changes

Apply configuration changes here

Outbound Route

Click on Outbound Routes(from the left menu)

Fill up the following info as below

Route name			:

Prefix 				:

Match pattern			:

Trunk seq For Mached routes	:

Apply configuration changes here

Inbound Routes

Click on Inbound Routes (from the left menu)

Select extensions as you want to use in your VOIP Operator.

IVR

Click on system recording (from the left menu) for record your voice from IP phone or any other extensions

Type your extensions number to record voice

And click on Go

Name of the recording		 :		welcome _voice